WaveArtist

ISA BUS
Command/Status Interface Description

PRELIMINARY

Rockwell International
Proprietary Information
No Dissemination or Use Allowed
Without Prior Written Permission

Signal Processing Products Development
Multimedia Communications Division

Rockwell Semiconductor Systems

Overview

Sound Blaster Emulation Interface

MPU-401 Interface

FM Interface

WaveArtist Command/Status Interface

WaveArtist Command/Status Registers

Interrupt Contention Considerations

Query and Diagnostic Commands

00h - System ID and Revision Request

01h - Available Function Request

02h - System Function Status

08h - C19 RAM Download

n09h - Run C19 Diagnostic

0Ah - Read C19 ROM Data Block

0Bh - Retrieve C19 Stack Pointer

0Ch - Assert C19 IRQ

0Dh - Read C19 Memory Location

07h - Write 16bit C19 Memory Locations

nn0Eh - Write C19 Memory Location

0Fh - Execute C19 Code at Given Location

Wave I/O Commands

10h - Set Sample Width/Format for Data Path to PC

11h - Set Number of Channels for Data Path to PC

12h - Set Sample Speed Time Constant for Data Path to PC

13h - Assign ADC Data Transfer to PC Path

14h - Set Sample Count for Block Data Transfer to PC

15h - Enable ADC Data Transfer to PC

16h - Pause ADC Data Transfer to PC

17h - Stop ADC Data Transfer to PC

18h - Resume ADC Data Transfer to PC

19h - Read Recording 16-bit Samples

20h - Set Sample Width/Format for Data Path from PC

21h - Set Number of Channels for Data Path from PC

22h - Set Sample Speed Time Constant for Data Path from PC

23h - Assign DAC Data Transfer from PC Path

24h - Set Sample Count for Block Data Transfer from PC

25h - Enable DAC Data Transfer from PC

26h - Pause DAC Data Transfer from PC

27h - Stop DAC Data Transfer from PC

28h - Resume DAC Data Transfer from PC

29h - Play Input Samples Directly

nn42h - Clear DMA Status Indicator(s)

Mixer and Volume Control Commands

nn30h - Read Mixer Control Word or Volume Setting

nn31h - Write New Volume Setting Pair

32h - Write New  Mixer Control Setting Pair

33h - Reset Mixer Settings

nn34h - Select Mono Record Source

Extended Interface Commands

nn50h - Select MIDI Output Path

nn51h - Set Number of L71 Wavetable Voices

nn60h -Diagnostic Read of L71

nn61h -Diagnostic Write to L71

64h - L71 Sample ROM Checksum Test

70h - Power Down Control

nn62h - Initiate DRAM Download

63h - Terminate DRAM Download

65h -Set Homac Decompression Filter Coefficient

APPENDIX A -- Windows Driver Support for Wave Table DRAM Download

DRV_RWA_DOWNLOAD Message

RWA_DLOADINFO Structure

File Download Example

# Overview

This document describes, from the orientation of the PC, the ISA Bus interfaces and their relative functions, for the WaveArtist product.  These interfaces are categorized according to the register mapping described in the WaveArtist Functional Specification -- Sound Blaster, MPU-401, FM (OPL3), and WaveArtist.  Each category is described as a separate section within this document.

# Sound Blaster Emulation Interface

The WaveArtist hardware implements a Sound Blaster Pro compatible  ISA bus interface.  Existing PC software compatible with this interface should function without modification.  No further description is necessary.

# MPU-401 Interface

The WaveArtist hardware  implements the Roland MPU-401 UART interface.  Existing software compatible with this interface should function without modification.  No further description is necessary.

# FM Interface

The WaveArtist hardware implements the Adlib compatible, 4-op register set for FM synthesis.  All PC software compatible with this interface should function without modification.  No further description is necessary.

# WaveArtist Command/Status Interface

## WaveArtist Command/Status Registers

The Command/Status registers exist on the ISA bus between the PC and the C19 and  are used to control Wave data I/O, mixer functions,  and WaveArtist extended functions such as downloading wave tables.  It is assumed that the base address of the register set is determined during bootstrap via PnP, or via user configuration options.  The registers are presented on the ISA bus according to the following register map:

| <Base+0> <Base+1> | CMDR | Command Register |
| --- | --- | --- |
| <Base+2> <Base+3> | DATR | Data Register |
| <Base+4> | CTLR | Control Register |
| <Base+5> | STATR | Status Register |
| <Base+6> | CTLX1R | Expansion Control 1 |
| <Base+7> | CTLX2R | Expansion Control 2 |
| <Base+8> <Base+9> | DATX1R | Expansion Data 1 |
| <Base+11> <Base+10> | DATX2R | Expansion Data 11 |

As shown, some of these registers are 16-bits wide and others are 8-bit registers.  The PC shall use 16-bit instructions to read/write 16-bit registers and 8-bit instructions to access the single byte registers.  The functions of the registers within this range are described in detail within the following table:

| Location, Size, Access, Default | Name | Meaning to C19 | Meaning to PC |
| --- | --- | --- | --- |
| <Base> + 0 16-bits Read/Write by both C19 and PC Def=None | WaveArtist Command /Response Register (CMDR) | The C19 reads all WaveArtist 16-bit commands entered into this register by the PC and may also write certain 16-bit command response/status data into this register for processing by the PC. Since, there are separate latches for read and write, CMDR can be thought of as two 16-bit registers, one for read and one for write. The C19 shall read and process the contents of CMDR after the PC writes the register. At the time the PC writes CMDR, the hardware automatically clears CMD_WE in the STATR register. When the C19 reads CMDR, the hardware sets CMD_WE back to 1, allowing another command. Certain commands may require the C19 to write a 16-bit quantity into the CMDR register as a response. In such cases, the C19 shall write the response into the register. This causes the hardware to set the CMD_RF bit in STATR to indicate to the PC that the response is available. Note that the time it takes for the C19 to generate a command response value in CMDR may be specific to the type of command issued originally by the PC. It is assumed that the C19 can only process one such response at a time in order to synchronize commands with responses. It is assumed that the C19 will not read a subsequent command entered into CMDR by the PC during the time it is processing a command that generates a response. Although the PC may enter such a subsequent command into the CMDR register, the CMD_WE bit would then remain off, thus preventing the PC from entering a third command, until the response is generated for the first command and placed in the output latch for delivery to the PC. | The PC shall use this register to issue 16-bit Wave I/O setup, mixer setup, and extended interface setup commands to the WaveArtist hardware, and to receive 16-bit responses for a subset of these commands. The PC may enter a command into CMDR only if the CMD_WE bit is set in the STATR register. The PC may poll this bit according to its own scheme; this may or may not be in response to a PC interrupt generated by the C19 as a result of the CMD_WEIE bit previously being set by the PC. When the PC writes a value into CMDR, it shall not attempt to write another command until the CMD_WE bit is set again, as the result of the C19 reading the previous data. For commands which require the C19 to send a response value through CMDR, the PC shall wait on CMD_RF as an indicator that it may read CMDR to get the response. This may or may not be in response to the CMD_RFIE interrupt. As a rule, the PC should not attempt to issue a command to the C19 via CMDR during the period while it is waiting for a response from a previous command. |
| <Base> + 2 16-bits Read/Write by both C19 and PC Def = None | WaveArtist Data Register (DATR) | The C19 reads 16-bit quantities entered into this register by the PC and may also write 16-bit quantities into this register for processing by the PC. This may be used for programmed I/O of wave data. The register works identically to the CMDR register except that the C19 will make no attempt to synchronize commands and responses -- thus the C19 will read DATR whenever the PC writes it and the C19 shall write DATR whenever it has data for the PC. The C19 shall read and process the contents of DATR after the PC writes the register. At the time the PC writes DATR, the hardware automatically clears DAT_WE in the STATR register. When the C19 reads DATR, the hardware sets DAT_WE back to 1, allowing another write by the PC. The C19 can initiate data transfer to the PC, via this register by writing the DATR register. When the C19 writes, the DAT_RF bit in STATR is set by the hardware to indicate to the PC that the data is available. | The PC shall use this register to send and receive 16-bit data values for parameters and/or programmed I/O. The PC may enter a value into DATR only if the DAT_WE bit is set in the STATR register. The PC may poll this bit according to its own scheme; this may or may not be in response to a PC interrupt generated by the C19 as a result of the DAT_WEIE bit previously being set by the PC. When the PC writes a value into DATR, it shall not attempt to write another command until the DAT_WE bit is set again, as the result of the C19 reading the previous data. For data sent to the PC through DATR, the PC shall wait on DAT_RF as an indicator that it may read DATR to get the response. This may or may not be in response to the DAT_RFIE interrupt. |
| <Base> + 4 8-bits Read/Write by both C19 and PC Bit 7 Def = OFF Bit 6 Def= OFF Bit 5 Def= OFF Bit 4 Def=OFF Bit 3 Def=OFF Bit 2 Def=OFF Bit 1 Def=OFF Bit 0 Def=OFF | WaveArtist Control Register (CTLR) CMD_WEIE CMD_RFIE DAT_WEIE DAT_RFIE RESET DMA1_IE (FLG1) DMA0_IE (FLG0) IRQ_ACK | This register contains control fields which the C19 reads to determine interrupt enable modes, and possibly other commanded modes as desired by the PC. Each bit is described in detail below: - If this bit is set, the C19 shall force a PC interrupt every time the CMD_WE bit transitions from OFF to ON. Note, if CMDR is already empty (and CMD_WE already set) at the time the PC sets CMD_WEIE, the C19 shall force the interrupt immediately. - If this bit is set, the C19 shall force a PC interrupt every time the CMD_RF bit transitions from OFF to ON. Note, if CMDR is already full (and CMD_RF already set) at the time the PC sets CMD_RFIE, the C19 shall force the interrupt immediately. - If this bit is set, the C19 shall force a PC interrupt every time the DAT_WE bit transitions from OFF to ON. Note, if DATR is already empty (and DAT_WE already set) at the time the PC sets DAT_WEIE, the C19 shall force the interrupt immediately. - If this bit is set, the C19 shall force a PC interrupt every time the DAT_RF bit transitions from OFF to ON. Note, if DATR is already full (and DAT_RF already set) at the time the PC sets DAT_RFIE, the C19 shall force the interrupt immediately. - This bit instructs the C19 to totally reset the WaveArtist hardware and place the ISA bus interface in a known initial state. Note, the PC must keep the RESET bit set until the reset operation is complete. It is recommended that the reset active period is at least 7ms or longer. RESET shall not take longer than 300 ms to complete. At completion all us defaulted, except CMDR contains hex 55AA.. - When set, this bit tells the C19 to force a PC interrupt in concert with asserting the DMA1 bit in the STATR register. This indicates that the number of samples specified for transfer before a status interrupt for the 16-bit DMA channel (DMA1) have been transferred according to the currently selected DMA mode. NOTE: this bit may have other uses in other non-DMA modes. If DMA has not been properly setup and initiated before enabling DMA1_IE, its behavior is currently undefined. - When set, this bit tells the C19 to force a PC interrupt in concert with asserting the DMA0 bit in the STATR register. This indicates that the number of samples specified for transfer before a status interrupt for the 8-bit DMA channel (DMA0) have been transferred according to the currently selected DMA mode. NOTE: this bit may have other uses in other non-DMA modes. If DMA has not been properly setup and initiated before enabling DMA0_IE, its behavior is currently undefined. Also note that this is the DMA channel associated with Sound Blaster Pro emulation: if necessary, this bit can be eliminated in favor of using controls in the Sound Blaster interface. It is included here at this time for clarity. - When toggled, this bit informs the WaveArtist Hardware that the PC has seen and acknowledges all pending interrupts asserted. In certain cases, the hardware may require that this bit be set by the PC before another PC interrupt can be generated. | This register contains control bits which the PC can use to control interrupt enables and other WaveArtist behavior: - The PC sets this bit whenever it wants to be interrupted on the CMDR register going empty. - The PC sets this bit whenever it wants to be interrupted on the CMDR register receiving a response from the C19. - The PC sets this bit whenever it wants to be interrupted on the DATR register going empty. - The PC sets this bit whenever it wants to be interrupted on the DATR register going full. - The PC sets this bit whenever it wants to place the WaveArtist hardware into its initial, known state. The PC, upon clearing the RESET bit, then should wait for the CMD_RF bit in the STATR register. After CMD_RF goes ON, the PC should read the CMDR register, and should expect the value 0x55AA. If this value is not read, the reset failed. This should happen within 300ms after reset is deactivated. This reset control bit can be used to return to normal operation after a power down command is issued. - For DMA, the PC uses this bit to enable its interrupt for the 16-bit DMA channel (DMA1). The PC should program the WaveArtist hardware, and the DMA controller for the proper size DMA buffers and interrupt count, as well as direction, before enabling this interrupt. NOTE: disabling this bit does not halt DMA transfer, but merely inhibits the PC interrupt associated with reaching a preprogrammed count of samples for each transfer. This bit may be used for other purposes in the future. - For DMA, the PC uses this bit to enable its interrupt for the 8-bit DMA channel (DMA0), which is also associated with Sound Blaster Pro emulation. The PC should program the WaveArtist hardware, and the DMA controller for the proper size DMA buffers and interrupt count, as well as direction, before enabling this interrupt. NOTE: disabling this bit does not halt DMA transfer, but merely inhibits the PC interrupt associated with reaching a preprogrammed count of samples for each transfer. This bit may be used for other purposes in the future. If necessary, the PC could use the Sound Blaster interface to control 8-bit DMA operation, but this bit is provided in the definition at this time for clarity. - The PC shall set this bit and then clear it before exiting its interrupt service routine. Regardless of how many actual reasons for the interrupt may have been serviced by the ISR, the IRQ_ACK bit shall be toggled only once at the end of the ISR. |
| <Base> + 5 8-bits Read Only by PC, Written by C19 Bit 7 Def=ON Bit 6 Def=OFF Bit 5 Def=ON Bit 4 Def=OFF Bit 3 Def=OFF Bit 2 Def=OFF Bit 1 Def=OFF Bit 0 Def=OFF | WaveArtist Status Register (STATR) CMD_WE CMD_RF DAT_WE DAT_RF IRQ_REQ DMA1 (FLG3) DMA0 (FLG2) SPARE | This register contains status bits which the C19 sets to allows the PC to determine, possibly in response to an interrupt, when specific events occur in the WaveArtist hardware/firmware. - The hardware sets this bit whenever it is capable of receiving a 16-bit command from the PC in the CMDR register. This bit is cleared by the hardware when the C19 reads a command entered into CMDR by the PC. For commands requiring responses in CMDR, the C19 should refrain reading CMDR in order to hold CMD_WE clear until the response has been written. - The hardware sets this bit whenever the C19 writes data into the CMDR register. The C19 shall refrain from writing additional information into CMDR until CMD_RF is cleared. - The hardware sets this bit whenever it is capable of receiving a 16-bit datum from the PC in the DATR register. This bit is cleared by the hardware when the C19 reads a value entered into DATR by the PC. DAT_WE is totally asynchronous to DAT_RF. - The hardware sets this bit whenever the C19 writes data into the DATR register. The C19 shall refrain from writing additional information into DATR until DAT_RF has been cleared - This bit is set by the C19 to indicate that an interrupt has been asserted. - This bit shall be set by the C19 whenever a preprogrammed number of 8-bit bytes has been transferred to or from PC memory on the 16-bit DMA channel. If DMA1_IE is enabled, the C19 shall force the concurrent interrupt to the PC. This bit may have other future uses. - This bit shall be set by the C19 whenever a preprogrammed number of 8-bit bytes has been transferred to or from PC memory on the 8-bit DMA channel. If DMA0_IE is enabled, the C19 shall force the concurrent interrupt to the PC. This bit may have other future uses. Note, this is the Sound Blaster Pro emulation’s DMA channel. If necessary, this bit can be eliminated in favor of using the Sound Blaster control/status interface, but it is included here for completeness. - This bit is currently unused. | The PC uses the status bits to determine which events are pending. It may or may not poll this register in response to interrupts. - The PC may enter 16-bit command data into the CMDR register only if CMD_WE is set. This may, or may not be a result of an interrupt. Note that CMD_WE shall default to ON. If the PC is not currently in a mode where commands are to be written to the CMDR register, then it must insure that it ignores CMD_WE unless and until it enters such a state. - The PC shall read the CMDR register to obtain response data upon seeing the CMD_RF bit set. This may or may not be a result of an interrupt. When the PC reads CMDR, the hardware clears CMD_RF. Note: in the normal case, if the PC is waiting for a response through CMDR, it shall not attempt to write another command until CMD_RF has changed, and the response has been read. This handshake shall be implemented to synchronize command/responses through CMDR. - The PC may enter a 16-bit value into the DATR register only if DAT_WE is set. This may, or may not be a result of an interrupt. Note that DAT_WE shall default to ON. If the PC is not currently in a mode where data is to be written to the DATR register, then it must insure that it ignores DAT_WE unless and until it enters such a state. - In certain modes, the PC may read the DATR register to obtain data upon seeing the DAT_RF bit set. This may or may not be a result of an interrupt. When the PC reads DATR, the hardware clears DAT_RF. Note that DAT_RF and DAT_WE are totally asynchronous and may occur at different times and rates. - The PC shall, upon seeing IRQ_REQ set, service the appropriate interrupt and then set the IRQ_ACK bit in the CTLR register to indicate that the request has been serviced. Note that the PC does not require the IRQ_REQ bit. - In DMA mode, the PC shall, upon seeing this bit set, assume that a preprogrammed number of bytes have been transferred to or from PC memory by the DMA controller on the 16-bit DMA channel. Normally, this number will be set to half of the DMA buffer size for this channel. The PC may or may not service this bit a result of the associated interrupt. - In DMA mode, the PC shall, upon seeing this bit set, assume that a preprogrammed number of bytes have been transferred to or from PC memory by the DMA controller on the 8-bit DMA channel, which is also the Sound Blaster Pro emulation channel. Normally, this number will be set to half of the DMA buffer size for this channel. The PC may or may not service this bit a result of the associated interrupt. - This bit is currently unused. |
| <Base> + 6 8-bits Read and Write by both PC and C19 | WaveArtist Expansion Control Register 1 (CTLX1R) | TBD | TBD |
| <Base> + 7 8-bits Read and Write by both PC and C19 | WaveArtist Expansion Control Register 2 (CTLX2R) | TBD | TBD |
| <Base> + 8 16-bits Read and Write by both PC and C19 | WaveArtist Expansion Data Register 1 (DATX1R) | TBD | TBD |
| <Base> + 10 16-bits Read and Write by both PC and C19 | WaveArtist Expansion Data Register 2 (DATX2R) | TBD | TBD |
| <Base> + 12 8-bits Read only by PC Bit 0 Def=OFF Bit 1 Def=OFF Bit 2 Def=OFF Bit3 Def=OFF Bit4 Def=OFF | WaveArtist Logical Device IRQ Status Register IRQSTAT Wave Artist Logical Device IRQ Sound Blaster Pro Logical Device IRQ MPU401 Logical Device IRQ Modem Logical Device IRQ CDROM Logical Device IRQ |  | - These IRQ status bits are active high status bits. They are set and reset automatically by the ASIC hardware to reflect the current state of each logical device IRQ lines to the ISA bus. |

## Interrupt Contention Considerations

As defined, the WaveArtist interface is capable of generating interrupts on the PC.  There are issues with interrupt contention which must be dealt with at the PC software level.  Since WaveArtist commands can be issued at either interrupt level or non-interrupt level, there must be a guarantee that  PC interrupt software does not attempt to  issue a command to the WaveArtist hardware during a period when PC non-interrupt software is in the midst of issuing a different command to WaveArtist.

At the present time, the only foreseen solution to this issue is to require that all PC software which talks to the WaveArtist hardware, must issue commands only with PC interrupts disabled; that is, in a critical section.  This guarantees that a pending ISR and non-ISR code will access the hardware only in a mutually exclusive manner.

## Query and Diagnostic Commands

Listed below are WaveArtist commands which can be used by the PC software to determine system capabilities and viability:

#### 00h - System ID and Revision Request

Cmd Number  written to CMDR by PC:	00h

Parameter written to CMDR by PC:	None.

Response written to CMDR by C19:	Word 1
	Low Byte  = Product ID (30 = ASCII “0” for last digit of 1158x). 
	High Byte = Code Revision (41 = ASCII “A”).

Word 2
	L71 Part Number (3030 = ASCII “00” for the last two digits of L71xx),  0000 if no L71 present.

Restrictions:	None.

Description:

This command may be used by the PC  to ask  the WaveArtist hardware/firmware to identify itself.  This can be used by the PC to support product differentiation in the future.

#### 01h - Available Function Request

Cmd Number  written to CMDR by PC:	01h

Parameter written to CMDR by PC:	None.

Response written to CMDR by C19:	2 Words.  First word is capability map -- see table below.  Second word indicates DRAM size as follows:  0080 indicates 4Mx16 DRAM; 00A0 indicates 1Mx16 DRAM; 00A8 indicates 256Kx16 DRAM.

Restrictions:	None.

Description:

This command may be used by the PC  to ask  the WaveArtist hardware/firmware which functions are available in the present configuration.  The returned  first word in CMDR is a bit map of available functions/attributes as follows:

| Bit Number | Function |
| --- | --- |
| 0 | These two bits indicate sample ROM size. 00=No ROM, |
| 1 | 01=One 2MB, 10=One 4MB, 11= Two 2MB |
| 2 | Spare |
| 3 | If set, Marge 2 present. |
| 4 | If set, 3D effects present. |
| 5 | If set, Treble/Bass Equalizer present. |
| 6 | If set, MPEG Audio Decoder Present |
| 7 | If set, Sample FIFO to WaveTable Engine present |
| 8 | Spare |
| 9 | Spare |
| 10 | Spare |
| 11 | Spare |
| 12 | Spare |
| 13 | Spare |
| 14 | Spare |
| 15 | Spare |

#### 02h - System Function Status

Cmd Number  written to CMDR by PC:	02h

Parameter written to CMDR by PC:	None.

Response written to CMDR by C19:	See Table Below.

Restrictions:	None.

Description:

This command may be used by the PC  to ask  the WaveArtist hardware/firmware which functions currently being utilized.  The returned word in CMDR is a bit map of used functions as follows  (NOTE: This command currently returns 0):

| Bit Number | Function |
| --- | --- |
| 0 | Spare |
| 1 | Spare |
| 2 | Spare |
| 3 | Spare |
| 4 | Spare |
| 5 | Spare |
| 6 | Spare |
| 7 | Spare |
| 8 | Spare |
| 9 | Spare |
| 10 | Spare |
| 11 | Spare |
| 12 | Spare |
| 13 | Spare |
| 14 | Spare |
| 15 | Spare |

#### 08h - C19 RAM Download

Cmd Number  written to CMDR by PC:	08h

Parameter written to CMDR by PC:	16-bit start address, followed by 8-bit data sequence until abort by including FF in the upper byte of the last input data or when address 27F is reached.

Response written to CMDR by C19:	None.

Restrictions:	Diagnostic, play at your own risk.

Description:

This command may be used by the PC to download data into the C19 RAM for downloaded execution or RAM test.  The address is followed by a sequence of bytes for download at that address.  Each write operation must happen after the PC sees CMD_WE.

#### n09h - Run C19 Diagnostic

Cmd Number  written to CMDR by PC:	n09h 	where “n” is a parameter included in the command word indicating which diagnostic function to run as follows:

n=0	All tests

n=1	ROM test only

n=2	RAM test only

n=3	Timer test only

Parameter written to CMDR by PC:	None.

Response written to CMDR by C19:	n=0 three words will be returned consecutively (0031(for 11581 or 0030 for 11580), 0080, and 008D)

n=1 one word will be returned (0031 for 11581 or 0030 for 11580)

n=2 one word will be returned (0080)

n=3 one word will be returned (008D)

Restrictions:

Description:

This command may be used by the PC to execute one or more diagnostic test routines for C19 ROM (code & SRC ROM table), C19 RAM, and/or timer checksum tests.  The result checksum is returned at the CMDR for the PC to read.  Checksum results are stored internally at C19 address 140=30 (ROM), 141=80 (RAM), and  142=8D (timer).

#### 0Ah - Read C19 ROM Data Block

Cmd Number  written to CMDR by PC:	0Ah

Parameter written to CMDR by PC:	16-bit C19 ROM start address.  PC may terminate dump by writing FF into CMDR at any time during the process.

Response written to CMDR by C19:	8-bit data bytes are written to CMDR one at a time until either address FFFF is reached, or the PC writes FF into CMDR.

Restrictions:	Diagnostic, play at your own risk.

Description:

This command may be used by the PC dump blocks of C19 ROM.  The PC should wait for CMD_RF between each  8-bit byte that is read from the C19.

#### 0Bh - Retrieve C19 Stack Pointer

Cmd Number  written to CMDR by PC:	0Bh

Parameter written to CMDR by PC:	None.

Response written to CMDR by C19:	Low Byte  = 8-bit C19 Stack pointer value

Restrictions:	None.

Description:

This diagnostic command can be used to probe the integrity of the c19 stack.

#### 0Ch - Assert C19 IRQ

Cmd Number  written to CMDR by PC:	0Ch

Parameter written to CMDR by PC:	None.

Response written to CMDR by C19:	None

Restrictions:	None.

Description:

This diagnostic command can be used to test the C19’s ability to cause an interrupt on the PC.  The IRQ_REQ bit in the STATR register should end up ON, with the PC experiencing the designated IRQ.  The PC can then toggle the IRQ_ACK bit in the CTLR register.

#### 0Dh - Read C19 Memory Location

Cmd Number  written to CMDR by PC:	0Dh

Parameter written to CMDR by PC:	16-bit C19 Memory Address.

Response written to CMDR by C19:	Low Byte contains the 8-bit contents of the referenced C19 memory address location.

Restrictions:	None.

Description:

This diagnostic command can be used by the PC program to read the contents of the C19s address space for debugging purposes.

#### 07h - Write 16bit C19 Memory Locations

Cmd Number  written to CMDR by PC:	07h

Parameter written to CMDR by PC:	First parameter is 16-bit C19 Memory address where the least significant data byte will be placed. Second is the data word to be placed in the two consecutive data bytes starting at the specified address.

Response written to CMDR by C19:	None

Restrictions:	None.

Description:

This diagnostic command can be used by the PC to write the C19s memory space (RAM) into two consecutive bytes. This command should be used in place of writing a signle byte at a time to set a 16 bit value such as jump vector for background auxilary task.

#### nn0Eh - Write C19 Memory Location

Cmd Number  written to CMDR by PC:	nn0Eh	-	where  “nn” is the 8-bit value to be written to C19 memory at the location specified by the parameter sent through CMDR.

Parameter written to CMDR by PC:	16-bit C19 Memory address where the “nn” component of the command word above is to be placed.

Response written to CMDR by C19:	None

Restrictions:	None.

Description:

This diagnostic command can be used by the PC to write the C19s memory space (RAM).  The data to be written is stored in the upper byte of the command word  “nn0E.”

#### 0Fh - Execute C19 Code at Given Location

Cmd Number  written to CMDR by PC:	0Fh

Parameter written to CMDR by PC:	16-bit C19 Memory Address.

Response written to CMDR by C19:	None.

Restrictions:	A bad address will cause unpredictable results.

Description:

This diagnostic command can be used by the PC program instruct the C19 to JMP to the location specified in the parameter sent through CMDR.

## Wave I/O Commands

Wave I/O can be controlled via either the Sound Blaster emulation interface or through the WaveArtist command/status registers described in this section.   WaveArtist can accommodate bi-directional Wave data throughput via DMA and possibly programmed I/O.   The following subsections list of all WaveArtist commands related to playing and recording Wave data:

#### 10h - Set Sample Width/Format for Data Path to PC

Cmd Number  written to CMDR by PC:	10h

Parameter written to CMDR by PC:	0 = 	8-bit signed PCM  (Default)
1 = 	16-bit signed PCM 
2 =	8-bit unsigned PCM

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Selection

Restrictions:	If used while the ADC path to the PC is fully active, the results are unpredictable -- samples will likely be lost, buffers will likely be cleared and reset to the initial count.

Description:

This command may be used by the PC  to set the ADC format to 8-bit or 16-bit samples for the data stream coming to the PC. Note that 8-bit data is the default setting.  When the 10h is entered into the CMDR register, the C19 reads the command and then expects the format parameter to be entered into CMDR as the next immediate value.  When this value is read, the C19 selects the chosen sample width/format for its current ADC mode (if legal).  The PC must write the command word first in CMDR, followed by the format selection parameter in CMDR.  In each case, the CMD_WE bit must be set before the PC writes.   The PC must accept a response from the C19 for this command via CMDR after it writes the parameter word, when CMD_RF is set.  The value read from CMDR by the PC indicates whether the selected sample format, has  been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

Note, acceptance of this command overrides the previous ADC format/width settings.

#### 11h - Set Number of Channels for Data Path to PC

Cmd Number  written to CMDR by PC:	11h

Parameter written to CMDR by PC:	1 = MONO  (Default)
2 = STEREO

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Selection

Restrictions:	If used while the ADC path to the PC is fully active, the results are unpredictable -- samples will likely be lost, buffers will likely be cleared and reset to the initial count.

Description:

This command may be used by the PC  to select MONO or STEREO for the ADC data stream coming to the PC.  When the 11h is entered into the CMDR register, the C19 must then take the next value written into the CMDR as the desired number of channels.  The PC must write the command word first in CMDR, followed by the parameter word. In each case, the CMD_WE bit must be set before the PC writes.   The PC must accept a response from the C19 for this command via CMDR after it writes the parameter, when CMD_RF becomes set.  The value read from CMDR by the PC indicates whether the selected number of channels has been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

Note, acceptance of this command overrides the previous ADC stereo/mono setting.

#### 12h - Set Sample Speed Time Constant for Data Path to PC

Cmd Number  written to CMDR by PC:	12h

Parameter written to CMDR by PC:	Desired sample rate expressed as an 16-bit value (K) as a parameter in CMDR according to the formula:

 K       =	 (65536*SampleRate)/44100
	32 bit calculation but only use the lower 16 bit of the 	result for the required parameter

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Rate Selection

Restrictions:	May be used to alter the sample speed on the fly.  However, such behavior would be somewhat precarious for the PC software, since the DMA interrupt rate, would be affected. Due to restrictected sample rate converter implementation changing sample speed on the fly may not be glitch free.

Description:

This command may be used by the PC  to set the ADC sample rate for the data stream coming to the PC. When the 12h is entered into the CMDR register, the C19 then expects the speed parameter to be sent. The desired sample rate for this is entered as a parameter in the CMDR register as described in the formula above (This may change at some date).   The PC must write the command word first in CMDR, followed by the data word in CMDR.  In each case, the CMD_WE bit must be set before the PC writes.   The PC must accept a response from the C19 for this command via CMDR after the parameter is written, when CMD_RF is set.  The value read from CMDR by the PC indicates whether the selected sample rate has been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

For the nominal speeds supported, the formula works out to the hex values in the following examples:

Sample Rate		  K

8000 KHz		2E71

11.025 KHz		4000

22.05 KHz		8000

44.1 KHz		0000

Note, acceptance of this command overrides the previous ADC sample speed setting.

#### 13h - Assign ADC Data Transfer to PC Path

Cmd Number  written to CMDR by PC:	13h

Parameter written to CMDR by PC:	0 = Use 8-bit DMA Channel  (Sound Blaster Pro DMA Channel)
1 = Use 16-bit DMA Channel
2=  Use programmed I/O mode

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Path Selection

Restrictions:	If the command is issued while ADC data is being routed to the PC, then the results are unpredictable.  PC should terminate the ADC path to the PC before issuing the command.   If the selection is already in use by the DAC path the selection is rejected.

Description:

This command may be used by the PC  to set the ADC path used for the data stream coming to the PC.  This can be either of the DMA channels, or the programmed I/O mode via the DATR register.  The default selection is the 16-bit DMA channel.

When the 13h is entered by the PC into CMDR, the C19 then assumes that the desired path selection (0, 1, or 2) shall be entered immediately (but after) into CMDR by the PC.  The PC writes only when CMD_WE is enabled.  The PC must accept a response from the C19 for this command via CMDR, when CMD_RF is set.  The value read from CMDR by the PC indicates whether the selected path has been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

Note, for programmed I/O mode, 8 or 16-bit data will be sent through the DATR register.  Each byte or word transferred will be read by the PC if and only if the DAT_RF bit has been set.  Note however, that in programmed I/O mode, the sample count (see below) associated with the transfer can still be used to generate interrupts less often than on every sample.

#### 14h - Set Sample Count for Block Data Transfer to PC

Cmd Number  written to CMDR by PC:	14h

Parameter written to CMDR by PC:	Number  (1-64K) of samples to transfer to the PC before asserting the status bit and interrupt (if enabled) associated with fulfilling the count.  See description.

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable count

Restrictions:	If used while the ADC channel is actively sending data to the PC, the results are unpredictable.

Description:

This command may be used by the PC  to set the “between interrupts” count used for the data stream coming to the PC.  This is a number between 1 and 64K and refers to the number of  samples sent to the PC.   For the purposes of this command, a sample is  an 8-bit byte or 16-bit word depending upon the currently selected sample width value. Note, other sample widths may be possible in the future.

The meaning of the sample count parameter is detailed below:

a.	In 8-bit MONO mode each sample is one byte received from the single ADC channel. A byte is available each sample period, thus the count is a byte count.  Note that if the 16-bit DMA channel is in use, each DMA cycle can accommodate two 8-bit sample transfers to the PC and the C19 must decrement its internal count twice per DMA cycle. If the 8-bit DMA channel is in use, each sample transfer to the PC occupies one DMA cycle.

b.	In 16-bit MONO mode each sample is a 16-bit word.  A 16-bit sample is sent each sample period ,  thus the count is a word count.  Note that if the 8-bit DMA channel is used, each 16-bit sample datum requires 2 DMA cycles;  the C19’s internal count however is decremented only once for each 2 DMA cycles.  If the 16-bit DMA channel is used in this mode, each sample item requires one DMA cycle, for which the C19’s internal count is decremented once.

c.	In 8-bit STEREO mode, 2 single byte samples are sent,  one from each channel, for the sample period.  However, the count provided to the C19 is still a total count of 8-bit samples per block, and thus is decremented by the C19 twice per sample period.  If the 8-bit DMA channel is in use, the C19 alternates the left and right channel samples for each DMA cycle transfer, decrementing its internal count once per DMA cycle.  If the 16-bit DMA channel is used,  the C19 packs both the left and right sample into each 16-bit DMA cycle transfer, decrementing its internal sample count twice per DMA cycle.

d.	In 16-bit STEREO mode, 2 single word samples are sent,  one from each channel, for the sample period.  The count provided to the C19 is a total count of 16-bit samples per block, and thus is decremented by the C19 twice per sample period, identically to 8-bit STEREO mode.  If the 8-bit DMA channel is in use however,  the C19 must use 4 DMA cycles each sample period, sending the low and high bytes of first the left channel, and then the low and high bytes of the right channel.  The C19’s internal count should be decremented for each two DMA cycle transfers, or twice per sample period as mentioned above.  If the 16-bit DMA channel is used,  the C19 sends first the left, and then the right channel 16-bit samples in 2 DMA cycle transfers, decrementing its count on each transfer.

.

The PC will most likely determine the count, sent to the C19 as a function of the sample size and speed.  Therefore, it is likely that the count will be sent every time the format and/or speed is changed.

The C19 must notify the PC when a block transfer is complete. It applies the  count parameter to the currently selected data path as follows:

1.	Notifying the PC at the end of  a sample block utilizing the 8-bit DMA Channel

If the 8-bit DMA (Sound Blaster) channel is selected, the C19 asserts DMA0 in the STATR register when the number of samples requested have been fully  transferred to the PC via the 8-bit DMA channel.  If and only if the DMA0_IE bit has been set by the PC,  the C19 additionally asserts the PC interrupt for this condition.

2.	Notifying the PC at the end of a sample block utilizing the 16-bit DMA Channel

If the 16-bit DMA channel is selected, the C19 asserts DMA1 in the STATR register when the number of samples requested have been transferred to the PC via the 16-bit DMA channel.  If and only if the DMA1_IE bit has been set by the PC,  the C19 additionally asserts the PC interrupt for this condition.

3.	Notifying the PC at the end of a sample block utilizing Programmed I/O

If the PIO mode is selected for ADC, the C19 asserts  which ever DMA status bit in STATR is currently unused by the DAC data path coming from the PC.  If the 8-bit DMA channel is being used for DAC data, then the C19 shall use  DMA1 in STATR, and the associated interrupt if DMA1_IE is selected,  to provide infrequent PIO interrupts.  If the 16-bit DMA Channel is being used for DAC data, then the C19 shall use DMA0 in a likewise manner.  These bits shall be allocated by the C19 similarly if the DAC path is setup for PIO.  If the DAC path is currently unused, then the PIO ADC path shall default to using the DMA1 bit and the associated interrupt if enabled.

This mechanism is designed to allow the PC to perform programmed I/O without requiring a PC interrupt on every sample datum.  When the PC receives the interrupt for the expired sample count, it will then get the data, 2 bytes  (16-bits) at a time via the DATR and DAT_RF mechanism.  If the sample width is 8-bits, this means that the C19 must always pack 2 samples into DATR per programmed I/O transfer (as if the 16-bit DMA channel were in use).

This mechanism implies that the interrupt, for programmed I/O should be generated only after the C19 has collected the full number of samples specified by the count in its internal memory, creating a software FIFO or “speed buffer” -- which implies that the C19 must buffer the data -- which further implies that for programmed I/O, the sample count will likely be quite small.  A better mechanism for PIO might employ hardware FIFOs between the PC and the WaveArtist hardware.

It is assumed that the specified count will be used by the C19 in an auto-reinitialize manner.  When the C19 fulfills a block transfer, it automatically reinitializes its count and is ready to begin the next block.  The PC can, at any time issue the PAUSE or TERMINATE commands, to stop the data flow.  Since the WaveArtist can receive these commands at any time, it is currently considered unnecessary for it to support a “single block” transfer request (though this could be added if desired).

When the 14h is entered by the PC into CMDR, the C19 then assumes that the desired sample item count shall be entered immediately into CMDR by the PC.  The PC writes only when CMD_WE is enabled.  The PC must accept a response from the C19 for this command via CMDR, when CMD_RF is set.  The value read from CMDR by the PC indicates whether the selected count has been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

#### 15h - Enable ADC Data Transfer to PC

Cmd Number  written to CMDR by PC:	15h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None (Current ADC setup is activated, whatever it is)

Restrictions:	Ignored if ADC already enabled.

Description:

This command may be used by the PC to begin ADC data transfer in the current selected data format and sample speed through the currently selected DMA or PIO data path to the PC.  Status bits and interrupts (if enabled) are set according to the mode.

#### 16h - Pause ADC Data Transfer to PC

Cmd Number  written to CMDR by PC:	16h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None (but ADC data will cease going to PC)

Restrictions:	Ignored if ADC already disabled.

Description:

This command is to be used by the PC to pause ADC data transfer to the PC.   The C19 stops sending data regardless of the format, or selected data path.  When the RESUME command is given, the C19 resumes sending the ADC data to the PC at the same position within the DMA block transfer; the sample item count is not reset by the PAUSE command.  This command should be used by the PC if the current DMA buffer is to be completed after issuing the RESUME command.

#### 17h - Stop ADC Data Transfer to PC

Cmd Number  written to CMDR by PC:	17h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None (but ADC data will cease going to PC)

Restrictions:	Ignored if ADC already disabled.

Description:

This command is to be used by the PC to stop ADC data transfer to the PC.   The C19 stops sending data regardless of the format, or selected data path.  When and if the ENABLE command is given, the C19 begins sending the ADC data to the PC after resetting its internal position relative to the sample item count.  This command should be used by the PC when the current DMA buffer is to be released.

#### 18h - Resume ADC Data Transfer to PC

Cmd Number  written to CMDR by PC:	18h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None (Current DAC setup is activated, whatever it is)

Restrictions:	Ignored if ADC already enabled.

Description:

This command may be used by the PC to resume ADC data transfer in the current selected data format and sample speed through the currently selected DMA or PIO data path to the PC.  Status bits and interrupts (if enabled) are set according to the mode.

#### 19h - Read Recording 16-bit Samples

Cmd Number  written to CMDR by PC:	19h

Parameter written to CMDR by PC:	None.

Response written to CMDR by C19:	Two 16-bit samples (left and right)

Restrictions:	Timing synchronization  is up to the PC.

Description:

This command may be used by the PC to read the SRC recording buffer before the FIFO directly.  It is up to the PC to keep reading with appropriate timing.  The sample rate is set by the set sample rate command.  The PC must issue the command for every sample set reading.

#### 20h - Set Sample Width/Format for Data Path from PC

Cmd Number  written to CMDR by PC:	20h

Parameter written to CMDR by PC:	0 = 	8-bit signed PCM  (Default)
1 = 	16-bit signed PCM 
2 =	8-bit unsigned PCM

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Selection

Restrictions:	If used while the DAC path from the PC is fully active, the results are unpredictable -- samples will likely be lost, buffers will likely be cleared and reset to the initial count.

Description:

This command may be used by the PC  to set the DAC format to 8-bit or 16-bit samples for the data stream coming from the PC.  Note that 8-bit data is the default setting.  When the 20h is entered into the CMDR register, the C19 reads the command and then expects the format parameter to be entered into CMDR as the next immediate value.  When this value is read, the C19 selects the chosen sample width/format for its current DAC mode (if legal).  The PC must write the command word first in CMDR, followed by the format selection parameter in CMDR.  In each case, the CMD_WE bit must be set before the PC writes.   The PC must accept a response from the C19 for this command via CMDR after it writes the parameter word, when CMD_RF is set.  The value read from CMDR by the PC indicates whether the selected sample format, has  been accepted by the WaveArtist hardware/firmware. The response from the C19 must occur within  TBD (5us).

Note, acceptance of this command overrides the previous DAC format/width settings.

#### 21h - Set Number of Channels for Data Path from PC

Cmd Number  written to CMDR by PC:	21h

Parameter written to CMDR by PC:	1 = MONO  (Default)
2 = STEREO

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Selection

Restrictions:	If used while the DAC path from the PC is fully active, the results are unpredictable -- samples will likely be lost, buffers will likely be cleared and reset to the initial count.

Description:

This command may be used by the PC  to select MONO or STEREO for the DAC data stream coming from the PC.  When the 21h is entered into the CMDR register, the C19 must then take the next value written into the CMDR as the desired number of channels.  The PC must write the command word first in CMDR, followed by the parameter word. In each case, the CMD_WE bit must be set before the PC writes.   The PC must accept a response from the C19 for this command via CMDR after it writes the parameter, when CMD_RF becomes set.  The value read from CMDR by the PC indicates whether the selected number of channels has been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

Note, acceptance of this command overrides the previous DAC stereo/mono setting.

#### 22h - Set Sample Speed Time Constant for Data Path from PC

Cmd Number  written to CMDR by PC:	22h

Parameter written to CMDR by PC:	Desired sample rate expressed as an 16-bit value (K) as a parameter in CMDR according to the formula .

K       =	 (65536*SampleRate)/44100
	32 bit calculation but only use the lower 16 bit of the 	result for the required parameter

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Rate Selection

Restrictions:	May be used to alter the sample speed on the fly.  However, such behavior would be somewhat precarious for the PC software, since the DMA interrupt rate, would be affected.

Description:

This command may be used by the PC  to set the DAC sample rate for the data stream coming from the PC. When the 22h is entered into the CMDR register, the C19 then expects the speed parameter to be sent. The desired sample rate for this is entered as a parameter in the CMDR register as described in the formula above (This may change at some date).   The PC must write the command word first in CMDR, followed by the data word in CMDR.  In each case, the CMD_WE bit must be set before the PC writes.   The PC must accept a response from the C19 for this command via CMDR after the parameter is written, when CMD_RF is set.  The value read from CMDR by the PC indicates whether the selected sample rate has been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

The following table shows the (hex) values that would be used for the common speeds.

Sample Rate		  K

8000 KHz		2E71

11.025 KHz		4000

22.05 KHz		8000

44.1 KHz		0000

Note, acceptance of this command overrides the previous DAC sample speed setting.

#### 23h - Assign DAC Data Transfer from PC Path

Cmd Number  written to CMDR by PC:	23h

Parameter written to CMDR by PC:	0 = Use 8-bit DMA Channel  (Sound Blaster Pro DMA Channel)
1 = Use 16-bit DMA Channel
2=  Use programmed I/O mode

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Path Selection

Restrictions:	If the command is issued while DAC data is being routed from the PC, then the results are unpredictable.  PC should terminate the DAC path from the PC before issuing the command.   If the selection is already in use by the ADC path the selection is rejected.

Description:

This command may be used by the PC  to set the DAC path used for the data stream coming from the PC.  This can be either of the DMA channels, or the programmed I/O mode via the DATR register.  The default selection is the 8-bit DMA channel  (Note the ADC and DAC default paths cannot use the same DMA channel).

When the 23h is entered by the PC into CMDR, the C19 then assumes that the desired path selection (0, 1, or 2) shall be entered immediately (but after) into CMDR by the PC.  The PC writes only when CMD_WE is enabled.  The PC must accept a response from the C19 for this command via CMDR, when CMD_RF is set.  The value read from CMDR by the PC indicates whether the selected path has been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

Note, for programmed I/O mode, 8 or 16-bit data will be transmitted through the DATR register.  Each byte or word transferred will be written by the PC if and only if the DAT_WE bit has been set.  Note however, that in programmed I/O mode, the sample count (see below) associated with the transfer can still be used to generate interrupts less often than on every sample.

#### 24h - Set Sample Count for Block Data Transfer from PC

Cmd Number  written to CMDR by PC:	24h

Parameter written to CMDR by PC:	Number  (1-64K) of samples to transfer from the PC before asserting the status bit and interrupt (if enabled) associated with fulfilling the count.  See description.

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable count

Restrictions:	If used while the DAC channel is actively fetching data from the PC, the results are unpredictable.

Description:

This command may be used by the PC  to set the “between interrupts” count used for the data stream coming from the PC.  This is a number between 1 and 64K and refers to the number of  samples to be transferred from the PC to the WaveArtist.   For the purposes of this command, a sample is  an 8-bit byte or 16-bit word depending upon the currently selected sample width value. Note, other sample widths may be possible in the future.

The meaning of the sample count parameter is detailed below:

a.	In 8-bit MONO mode each sample is one byte sent to  the single DAC channel. A byte is sent each sample period, thus the count is a byte count.  Note that if the 16-bit DMA channel is in use, each DMA cycle can accommodate two 8-bit sample transfers from the PC and the C19 must decrement its internal count twice per DMA cycle. If the 8-bit DMA channel is in use, each sample transfer from the PC occupies one DMA cycle.

b.	In 16-bit MONO mode each sample is a 16-bit word.  A 16-bit sample is sent to the single DAC channel each sample period ,  thus the count is a word count.  Note that if the 8-bit DMA channel is used, each 16-bit sample datum requires 2 DMA cycles;  the C19’s internal count however is decremented only once for the 2 DMA cycles.  If the 16-bit DMA channel is used in this mode, each sample transfer from the PC requires one DMA cycle, for which the C19’s internal count is decremented once.

c.	In 8-bit STEREO mode, 2 single byte samples are sent,  one on each DAC channel, for the sample period.  However, the count provided to the C19 is still a total count of  8-bit samples per block, and thus is decremented by the C19 twice per sample period.  If the 8-bit DMA channel is in use, the PC alternates the left and right channel samples for each DMA cycle transfer to the C19, which should decrement its internal count once per DMA cycle.  If the 16-bit DMA channel is used,  the PC packs both the left and right sample into each 16-bit DMA cycle transfer to the C19,  which should decrement its internal sample count twice per DMA cycle.

d.	In 16-bit STEREO mode, 2 single word samples are sent,  one for each DAC channel, for the sample period.  The count provided to the C19 is a total count of 16-bit samples per block, and thus is decremented by the C19 twice per sample period, identically to 8-bit STEREO mode.  If the 8-bit DMA channel is in use however,  the PC must use 4 DMA cycles each sample period, sending the low and high bytes of first the left channel, and then the low and high bytes of the right channel to the C19.  The C19’s internal count should be decremented for each two DMA cycle transfers from the PC, or twice per sample period as mentioned above.  If the 16-bit DMA channel is used,  the C19 receives first the left, and then the right channel 16-bit samples in 2 DMA cycle transfers, decrementing its count on each transfer from the PC.

.

The PC will most likely determine the count, sent to the C19 as a function of the sample size and speed.  Therefore, it is likely that the count will be sent every time the format and/or speed is changed.

The C19 must notify the PC when a block transfer is complete. It applies the  count parameter to the currently selected data path as follows:

1.	Notifying the PC at the end of  a sample block utilizing the 8-bit DMA Channel

If the 8-bit DMA (Sound Blaster) channel is selected, the C19 asserts DMA0 in the STATR register when the number of samples requested have been fully  transferred from the PC via the 8-bit DMA channel.  If and only if the DMA0_IE bit has been set by the PC,  the C19 additionally asserts the PC interrupt for this condition.

2.	Notifying the PC at the end of a sample block utilizing the 16-bit DMA Channel

If the 16-bit DMA channel is selected, the C19 asserts DMA1 in the STATR register when the number of samples requested have been transferred from the PC via the 16-bit DMA channel.  If and only if the DMA1_IE bit has been set by the PC,  the C19 additionally asserts the PC interrupt for this condition.

3.	Notifying the PC at the end of a sample block utilizing Programmed I/O

If the PIO mode is selected for DAC, the C19 asserts  which ever DMA status bit in STATR is currently unused by the ADC data path going to the PC.  If the 8-bit DMA channel is being used for ADC data, then the C19 shall use  DMA1 in STATR, and the associated interrupt if DMA1_IE is selected,  to provide infrequent PIO interrupts for DAC.  If the 16-bit DMA Channel is being used for ADC data, then the C19 shall use DMA0 in a likewise manner.  These bits shall be allocated by the C19 similarly if the ADC path is setup for PIO.  If the ADC path is currently unused, then the PIO DAC path shall default to using the DMA1 bit and the associated interrupt if enabled.

This mechanism is designed to allow the PC to perform programmed I/O without requiring a PC interrupt on every sample datum.  When the PC receives the interrupt for the expired sample count, it will then send the data, 2 bytes  (16-bits) at a time via the DATR and DAT_WE mechanism.  If the sample width is 8-bits, this means that the PC must always pack 2 samples into DATR per programmed I/O transfer (as if the 16-bit DMA channel were in use), and the C19 must read the 2 samples from DATR.

This mechanism implies that the interrupt, for programmed I/O should be generated only when the C19 has collected enough buffer space to contain the full number of samples specified by the count in its internal memory, creating a software FIFO or “speed buffer” -- which implies that the C19 must buffer the transmitted data -- which further implies that for programmed I/O, the sample count will likely be quite small.  A better mechanism for PIO might employ hardware FIFOs between the PC and the WaveArtist hardware.

It is assumed that the specified count will be used by the C19 in an auto-reinitialize manner.  When the C19 fulfills a block transfer, it automatically reinitializes its count and is ready to begin the next block.  The PC can, at any time issue the PAUSE or TERMINATE commands, to stop the data flow.  Since the WaveArtist can receive these commands at any time, it is currently considered unnecessary for it to support a “single block” transfer request (though this could be added if desired).

When the 24h is entered by the PC into CMDR, the C19 then assumes that the desired sample item count shall be entered immediately into CMDR by the PC.  The PC writes only when CMD_WE is enabled.  The PC must accept a response from the C19 for this command via CMDR, when CMD_RF is set.  The value read from CMDR by the PC indicates whether the selected count has been accepted by the WaveArtist hardware/firmware.  The response from the C19 must occur within  TBD (5us).

#### 25h - Enable DAC Data Transfer from PC

Cmd Number  written to CMDR by PC:	25h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None (Current DAC setup is activated, whatever it is)

Restrictions:	Ignored if DAC already enabled.

Description:

This command may be used by the PC to begin DAC data transfer in the current selected data format and sample speed through the currently selected DMA or PIO data path from the PC.  Status bits and interrupts (if enabled) are set according to the mode.

#### 26h - Pause DAC Data Transfer from PC

Cmd Number  written to CMDR by PC:	26h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None (but DAC data will cease going from PC)

Restrictions:	Ignored if DAC already disabled.

Description:

This command is to be used by the PC to pause DAC data transfer from the PC.   The C19 stops sending data regardless of the format, or selected data path.  When the RESUME command is given, the C19 resumes sending the DAC data from the PC at the same position within the DMA block transfer; the sample item count is not reset by the PAUSE command.  This command should be used by the PC if the current DMA buffer is to be completed after issuing the RESUME command.

#### 27h - Stop DAC Data Transfer from PC

Cmd Number  written to CMDR by PC:	27h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None (but DAC data will cease going from PC)

Restrictions:	Ignored if DAC already disabled.

Description:

This command is to be used by the PC to stop DAC data transfer from the PC.   The C19 stops sending data regardless of the format, or selected data path.  When and if the ENABLE command is given, the C19 begins sending the DAC data from the PC after resetting its internal position relative to the sample item count.  This command should be used by the PC when the current DMA buffer is to be released.

#### 28h - Resume DAC Data Transfer from PC

Cmd Number  written to CMDR by PC:	28h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None (Current DAC setup is activated, whatever it is)

Restrictions:	Ignored if DAC already enabled.

Description:

This command may be used by the PC to resume DAC data transfer in the current selected data format and sample speed through the currently selected DMA or PIO data path from the PC.  Status bits and interrupts (if enabled) are set according to the mode.  This command differs from the ENABLE command in that it instructs the C19 NOT to issue a “dummy” read to force an initial DMA request -- the RESUME assumes that a valid DMA sample is in the C19s DMA read register, and this sample is used rather than discarded.

#### 29h - Play Input Samples Directly

Cmd Number  written to CMDR by PC:	28h

Parameter written to CMDR by PC:	16-bit left, then 16-bit right sample

Response written to CMDR by C19:	None

Restrictions:	Diagnostic

Description:

This command may be used by the PC play samples direct to the SRC playback FIFO.  The PC must issue the command for every sample set.

#### nn42h - Clear DMA Status Indicator(s)

Cmd Number  written to CMDR by PC:	nn42h   -	where “nn” is the upper byte of the 16-bit value written to CMDR.  In this case, the upper byte is used to hold a parameter for the command which is an image of the DMA0 and DMA1 status bits in STATR identifying which DMA channel(s) are to be “acked.”   The legal values of  “nn42” are defined as follows:

0142h	-	Acknowledge that a pending DMA0 status (in the STATR register) has been fully dealt with and should be cleared by the hardware/firmware

0242h	-	Acknowledge that a pending DMA1 status (in the STATR register) has been fully dealt with and should be cleared by the hardware/firmware

0342h	-	Acknowledge that  pending DMA0 and DMA1 status bits  (in the STATR register) have been fully dealt with and should be cleared by the hardware/firmware

Parameter written to CMDR by PC:	This command does not write a separate parameter into CMDR, but the selection parameter is stored in the upper byte of the command request as shown above.

Response written to CMDR by C19:	None

Restrictions:	PC should refrain from using illegal parameters in the upper byte of the command word.  The C19 should ignore illegal command words.

Description:

This command is to be used by the PC to acknowledge that  pending DMA0 and/or DMA1 status bits (in the STATR register) have been fully dealt with and should be cleared by the hardware/firmware.  Until this command is issued, the C19 shall refrain from attempting to assert the DMA0  and/or DMA1 statuses again.  This command should be sent  by the PC AFTER the IRQ_ACK bit is toggled, for a pending DMA interrupt.  This allows the C19 to “find out” when its interrupt has been acknowledged without needing to poll the IRQ_REQ bit in the STATR register.  Since the C19 is already looking for commands via CMDR, this relieves C19 processing requirements during high speed DMA data transfers.  The ability to acknowledge both the 8-bit and 16-bit channel interrupts simultaneously allows the PC to service both interrupts during a single interrupt service routine invocation.  This means that the PC may be in the midst of servicing one interrupt, say DMA0, and then if the DMA1 bit goes ON, the PC can loop back and service this one as well, without exiting its ISR.  Upon completion, the PC ISR can “ack” both interrupts, thus preventing the C19 from raising the IRQ again.

## Mixer and Volume Control Commands

#### nn30h - Read Mixer Control Word or Volume Setting

Cmd Number  written to CMDR by PC:	nn30h   -	where “nn” is the upper byte of the 16-bit value written to CMDR.  In this case, the upper byte is used to hold a parameter for the command which is an index selecting which mixer or volume setting is to be returned as the response to the command.   The legal values of  “nn30” are defined as follows:

0030h   to  0930h	-	Return  one Mixer control word  in the range from 1 through 10.  The upper byte is the 4-bit 10495 control word address minus one.  Example:  0730h means return mixer control word 8.

0A30h	-	Return Left PCM channel volume word.

0B30h	-	Return Right PCM channel volume word.

0C30h	-	Return Left FM channel volume word.

0D30h	-	Return Right FM channel volume word.

0E30h	-	Return Left Wave Table channel volume word.

0F30h	-	Return Right Wave Table channel volume word.

1030h	-	Return left PCM expansion channel output volume word.

1130h	-	Return right PCM expansion channel output volume word..

1230h	-	Return left FM expansion channel output volume word.

1330h	-	Return right FM expansion channel output volume word.

Parameter written to CMDR by PC:	This command does not write a separate parameter into CMDR, but the index parameter is stored in the upper byte of the command request as shown above.

Response written to CMDR by C19:	Current Settings of  the Specified Mixer Control , expansion volume, PCM, FM, or Wave Table Volume word as specified in the request.   The formats of the ten Mixer control words are described in the WAFA Control Bit Definition table below.  The format of the Volume words is a 15-bit unsigned scalar (0 - 7FFFh).

NOTE: if the control word parameter is out of range, the C19 should return a 16-bit response of  FFFFh to indicate failure of the command.  In all  cases, a response MUST be issued by the C19 for this command.

Restrictions:	To avoid difficulty, the host code should never issue an out of range command.

Description:

This command is to be used by the PC to determine the current mixer and/or volume settings assumed in the hardware.  To save processing time on the C19, the requests for Mixer settings and Volume settings have been combined into this single “nn30” command and the requested selection is encoded in the upper byte of the command itself, which is placed into CMDR by the PC.  This is described above.   The contents of the responses vary according to the control word being interrogated.  The Volume words are 15-bit values, between 0 and 7FFFh.  The mixer control words take the following  general format:

The following WAFA Control Bit Definition table details the specific Mixer control words used in the WaveArtist product:

| 4 bit Address | 11 Bit Contents | Function | Logic Level |
| --- | --- | --- | --- |
| 1 | Left_Line_Mixer_Gain [10:6] Left_Aux1_Mixer_Gain [5:1] Left_Line_mute [0] | +12 to -33.0 dB and mute +12 to -33.0 dB and mute mute left line output | 11111 == +12dB 0==mute 11111 == +12dB 0==mute 0 == mute |
| 2 | Left_Aux2_Mixer_Gain [10:6] Right_2_Left_Mic_Gain [5:1] Mono_Mute [0] | +12 to -33.0 dB and mute +12 to -33.0 dB and mute mute mono line output | 11111 == +12dB 0==mute 11111 == +12dB 0==mute 0 == mute |
| 3 | Left_Mic_Mixer_Gain [10:6] Left_Mic_Gain [5:4] Left_MIxer_Gain [3:1] Dither_disable [0] | +12 to -33.0 dB and mute 0, 10 or +20dB 3dB step Mixer attenuation ADC dither disable | 11111 == +12dB 0==mute 11 == 20dB 0 ==mute 111 == 0dB 0== mute 0 == enable |
| 4 | Left_Mixer_Input_Select [10:4] Left_Rx_Filter_Gain [3] Left_ADC_Gain [2:0] | mux control to mixer 0 or +12dB gain 0 to 10.5dB in 1.5dB steps | see Mixer Input Control 1 == 12dB 0== 0dB 111 == 10.5dB |
| 5 | Right_Line_Mixer_Gain [10:6] Right_Aux1_Mixer__Gain [5:1] Right_Line_Mute [0] | +12 to -33.0 dB and mute +12 to -33.0 dB and mute mute right line output | 11111 == +12dB 0==mute 11111 == +12dB 0==mute 0 == mute |
| 6 | Right_Aux2_Mixer_Gain [10:6] Left_2_Right_Mic_Gain [5:1] test_1_bitout [0] | +12 to -33.0 dB and mute +12 to -33.0 dB and mute send ADC 1bit out to pad | 11111 == +12dB 0==mute 11111 == +12dB 0==mute 1 == enable |
| 7 | Right_Mic_Mixer_Gain [10:6] Right_Mic_Gain [5:4] Right_Mixer_Gain [3:1] Right_Mixer_Bypass [0] | +12 to -33.0 dB and mute 0, 10 or +20dB 3dB step Mixer attenuation bypass the mixer on DAC | 11111 == +12dB 0==mute 11 == 20dB 0==mute 111 == 0dB 0==mute 1 == bypass |
| 8 | Right_Mixer_Select [10:4] Right_Rx_Filter_Gain [3] Right_ADC_Gain [2:0] | mux control to mixer 0 or +12dB gain setting 0 to 10.5dB in 1.5dB steps | see Mixer Input Control 1 == +12dB 0== 0dB 111== 10.5dB |
| 9 | Mono_Mixer_Gain [10:6] Right_ADC_Mux_Select [5:3] Left_ADC_Mux_Select [2:0] | +12 to -33.0 dB and mute select 1 of 5 inputs to ADC select 1 of 5 inputs to ADC | 11111 == +12dB 0==mute 000==AGND 011==AUX2 001==MIXER 100==AUX1 010==LINE 101== MIC |
| 10 | Left_LB_ADC_2_DAC [10] Left_line_out [9:8] Right_LB_ADC_2_DAC [7] ADC_Chop_DIS [6] Txfilter_Chop_Dis [5] Offset_Cancel_Disable [4] Enable_serial _test_out [3] Left_LB_DAC_2_ADC_bar [2] Right_LB_DAC_2_ADC_bar [1] Over_Sampling_Ratio [0] | loop back ADC to DAC Left line output control loop back ADC to DAC serial output of control register loop back DAC to ADC loop back DAC to ADC choose 128 or 256 *Note(1) | 1== active 00 == L_mixer 10 == L_DAC 01 == R_mixer 11 == R_DAC 1 == active 0 == enable 0 == enable 0 == enable 1 == enable 0 == active 0 == active 1 == 256 0 == 128 |

Note (1)      although the sampling rate can be changed from 128 to 256, the external clock rate remains the same

Mixer Input Control:    This 7 bit control word controls the mux to the mixer. Each bit is an input enable, high true. Bit order is as follows: <MSB> Tx_filter, Mono, cross channel Mic, Mic ,Aux1, Aux2, Line <LSB>
Power-up:  All mute signals are defaulted to 0 (active) and all gain settings are defaulted to 0 (mute)

#### nn31h - Write New Volume Setting Pair

Cmd Number  written to CMDR by PC:	nn31h   -	where “nn” is the upper byte of the 16-bit value written to CMDR.  In this case, the upper byte is used to hold a parameter for the command which is an index selecting which volume setting is to be changed.  The legal values of  “nn31” are defined as follows:

0031h	-	Change Left and Right PCM (wave out) volume settings.

0131h	-	Change Left and Right FM (OPL3 MIDI out) volume settings.

0231h	-	Change Left and Right Wave Table (MPU-401 MIDI out) volume settings.

0331h	-	Change Left and Right PCM expansion channel output volume settings.

0431h	-	Change Left and Right FM expansion channel output volume settings.

.

Parameters written to CMDR by PC:	For this command, there are two separate 16-bit parameters sent through the CMDR register , each requiring a separate CMD_WE validation step.  The PC must always send both parameters.  They are:

Parm1	-	A 15-bit value in the range 0-7FFFh to be used by the C19 to set a new LEFT channel volume setting for the requested function.

Parm2	-	A 15-bit value in the range 0-7FFFh to be used by the C19 to set a new RIGHT channel volume setting for the requested function.

Response written to CMDR by C19:	This command currently has no response through CMDR.

Restrictions:	None.

Description:

This command is to be used by the PC to affect output volumes for Wave, FM, or Wave Table playback.  The PC always sends a LEFT and RIGHT setting even if only one channel is being adjusted by the user.

#### 32h - Write New  Mixer Control Setting Pair

Cmd Number  written to CMDR by PC:	32h

Parameters written to CMDR by PC:	For this command, there are two separate 16-bit parameters sent through the CMDR register , each requiring a separate CMD_WE validation step.   Each pair of parameters must represent a logical pair of  Mixer control words  which the C19 will send to the 10495 as follows:

Pair 1	-	In this case the first parameter is Mixer control word 1 and the second parameter is Mixer control word 5.

Pair 2	-	In this case the first parameter is Mixer control word 2 and the second parameter is Mixer control word 6.

Pair 3	-	In this case the first parameter is Mixer control word 3 and the second parameter is Mixer control word 7.

Pair 4	-	In this case the first parameter is Mixer control word 4 and the second parameter is Mixer control word 8.

Pair 5	-	In this case the first parameter is Mixer control word 9 and the second parameter is Mixer control word 10.

Response written to CMDR by C19:	None, currently.

Restrictions:	The host software should send only legal parameter pairs for this command.  The C19 should discard any illegal parameters.

Description:

This command is to be used by the PC to change the current mixer settings assumed in the hardware. Each parameter is a new 10495 mixer control word.  Even though each control word contains its address field,  the control words are always sent to the C19 in pairs corresponding mostly to logical “left/right” control words (1 and 5,  2 and 6,  3 and 7,  4 and 8,  9 and 10).  This is true even if only one control word is undergoing a change.  The “write” Mixer control word format is shown below:

#### 33h - Reset Mixer Settings

Cmd Number  written to CMDR by PC:	33h

Parameter written to CMDR by PC:	None

Response written to CMDR by C19:	None, currently.

Restrictions:	None.

Description:

This command is to be used by the PC to set all Mixer and Volume controls to their system default values.  These values are in order from nn=00 to 13 with the command nn30 reading as follows:

nn	Mixer(110495) Settings		nn	     Volume (ASIC) Settings

00		1801			0A		2413

01		2800			0B		2413

02		381C			0C		2413

03		4FF0			0D		2413

04		5801			0E		2413

05		6800			0F		2413

06		781C			10		0000

07		8FF0			11		0000

08		9805			12		0000

09		A836			13		0000

#### nn34h - Select Mono Record Source

Cmd Number  written to CMDR by PC:	nn34h   -	where “nn” is the upper byte of the 16-bit value written to CMDR.  In this case, the upper byte is used to hold a parameter for the command selects which channel is to be used as the recording source from a stereo source when making a mono recording, as follows:

0034h	-	Use the Left input source of the currently selected recording source to pass through for mono Wave Input.

0134h	-	Use the Right input source of the currently selected recording source to pass through for mono Wave Input.

Parameter written to CMDR by PC:	None, but parameter is in command word above.

Response written to CMDR by C19:	None, currently.

Restrictions:	Command should be legal.  C19 should ignore illegal commands.

Description:

This command is to be used by the PC select the source (left or right) used for making a single channel recording.  At some future time, it might be nice to incorporate a setting which digitally mixes both the left and right sources to make such a recording.

## Extended Interface Commands

#### nn50h - Select MIDI Output Path

Cmd Number  written to CMDR by PC:	nn50h   -	where “nn” is the upper byte of the 16-bit value written to CMDR.  In this case, the upper byte is used to hold a parameter for the command which is used by the C19 to determine where to route MIDI information coming from the PC through the MPU-401 interface.  The legal values of  “nn50” are defined as follows:

0050h	-	Route MIDI data to the external UART.

0150h	-	Route MIDI data to the WaveArtist Wave Table hardware.

0250h	-	Route MIDI data to both the WaveArtist Wave Table hardware and the external UART.  This is the default.

Parameters written to CMDR by PC:	None, except for the path select parameter in the upper byte of the command word.

Response written to CMDR by C19:	1 =  OK
0 = Unacceptable Path Selection or (more likely) the current hardware configuration does not support the requested path selection.  For instance, the C19 should respond “0” if the PC requests routing to the Wave Table hardware in a system in which there is no Wave Table hardware available.

Restrictions:	None.

Description:

This command may be used by the PC to switch the MIDI output destination between Wave Table and external UART (or both).  Processing should be similar to what the C19 does for the MPU-401 multi-port command (EB).  The Path selection must remain in effect until either a POR, hard reset command, or another set path command.  The default selection for this command shall be to route data to both the Wave Table and the external UART (or only the UART if there is no Wave Table hardware).

#### nn51h - Set Number of L71 Wavetable Voices

Cmd Number  written to CMDR by PC:	nn51h   -	where “nn” is the number of L71 Wavetable voices.

Parameter written to CMDR by PC:	In CMD byte above.

Response written to CMDR by C19:	None.

Restrictions:	Diagnostic.

Description:

This command may be used by the PC set the number of L71 Wavetable voices.  The upper byte of the command word contains the parameter.  This command will write to L71 byte A (C19 address 4A) to set the number of voices.

#### nn60h -Diagnostic Read of L71

Cmd Number  written to CMDR by PC:	nn60h   -	where “nn” determines what is read as follows:

nn=0	L71 DR

nn=1	L71 DI

nn=2	L71 CR

nn=3	L71 CI

nn=4	L71 single bank RAM

nn=5	L71 IO

nn=6	Homac registers

nn=7	Homac sample memory

nn=8	L71 IAR

Parameters written to CMDR by PC:	Commands 0060h - 0660h require a single address word parameter.

Command 0760h requires two address words (first is upper 8bit and second is lower 16bit) parameters.

Command 0860h does not require any parameter.

Response written to CMDR by C19:	Commands 0060h - 0560h, 0760h and 0860h return a single word data.

Command 0660h returns two data words. First is the upper word and second is the lower word.

Restrictions:	L71 diagnostic memory read access only.

Description:

This command may be used by the PC to perform a diagnostic read of the L71 RAM, IO, IAR, Homac registers and sample memory access including Marge2 registers.

#### nn61h -Diagnostic Write to L71

Cmd Number  written to CMDR by PC:	nn61h   -	where “nn” determines what to write as follows:

nn=0	L71 DR

nn=1	L71 DI

nn=2	L71 CR

nn=3	L71 CI

nn=4	L71 single bank RAM

nn=5	L71 IO

nn=6	Homac registers

nn=7	Homac sample memory

nn=8	L71 IAR

Parameters written to CMDR by PC:	Commands 0061h - 0561h require two parameters. First parameter is the address word and then next is the data word.

Command 0661h requires three parameters. First parameter is the address word. Second parameter is the upper data word. and the third is the lower data word.

Command 0761h requires three parameters. First parameter word is the upper 8bit (right justified) of the sample memory address. Second pareameter is the lower 16 bit of the sample memory address. Third parameter is the data word.

Command 0861h requires one data word parameter.

Response written to CMDR by C19:	None.

Restrictions:	Diagnostic write access to L71xx related registers. Must not be issued when L71xx

Description:

This command may be used by the PC to perform a diagnostic write to the L71 RAM, IO, IAR,  Homac registers, and sample memory including Marge2 registers. The primary usage of this command is to set the volume control for L71 A/D effects path (CR 49 and CR 4A), wavetable synthesis output volume (CI 3B and 3C), L71 effects control bits (IO 02 upper byte) and treble & bass equalizer setting (IO 04 lower byte). The L71 effects and treble & bass control bits definition are as follows:

L71 Effects Control (L71 mapped to C19 address as byte 45). If the L71 IO diagnostic command is used, note that you will have to use L71 word address 2 upper byte and the bits indicated below are in 16 bit word format.

Bit		Function

8		1 = Route 11581 expansion output to Homac FIFO, 0= Mix 11581 expansion output with Homac    		output

9		1 = Apply 3D spatializer effect processing before sending output to 11581, 0 = No spatializer effect

A		1 = Wavetable voice stereo pair1 for Marge2, 0 = stereo pair 0

B		Nothing

C		1 =  Enable treble & bass equalizer, 0 = Disable treble & bass equalizer

L71 Effects control bits definition (continued)

Bit		Function

D		Reserved for internal use. Do not attempt to change the state of this bit. 1 = Highspeed seral 			interface selected, 0 = Standard MIDI interface.

E		1 =  Enable Reverb, 0 = Disable reverb

F		1 = Enable Chorus, 0 = Disable chorus

Treble & bass equalizer control bits (lower nibble control the bass equalizer and upper nibble controls the treble equalizer)

Bit		Function

2 - 0 		0	Bass +14 dB

1	Bass +10.5 dB

2	Bass +7 dB

3	Bass + 3.5 dB

4	Bass - 3.5 dB

5	Bass - 7 dB

6	Bass -10.5 dB

7	Bass - 14 dB

6 - 4		0	Treble + 14 dB

1	Treble +10.5 dB

2	Treble + 7 dB

3	Treble + 3.5 dB

4	Treble - 3.5 dB

5	Treble - 7 dB

6	Treble - 10.5 dB

7	Treble - 14 dB

3 and 7	Not used

#### 64h - L71 Sample ROM Checksum Test

Cmd Number  written to CMDR by PC:	64h

Parameters written to CMDR by PC:	None.

Response written to CMDR by C19:	Sample ROM Checksum comes back in CMDR after CMD_RF

2MB ROM Sound Set returns 2208h.

Restrictions:	The PC must perform a C19 reset after this command. The L71 will go through the entire 1M word addresses and calculate the checksum. If smaller ROM (512x16) is present the checksum result will not matched the header because of the wraparound of addressing. The expected result of 512Kx16 RM is TBD.

Description:

This command may be used by the PC to reset the L71 and then request the L71 to perform a sample ROM checksum test.

#### 70h - Power Down Control

Cmd Number  written to CMDR by PC:	70h

Parameters written to CMDR by PC:	None.

Response written to CMDR by C19:	None

Restrictions:	No commands should be issued subsequent to this command until C19 reset command issued to get the chipset out of the shut down state.

Description:

This command may be used by the PC to do a low powrer shut down of the chip set. 10495 and L71xx will be held in reset and the 1158x will shut off all of the clocks and C19. The system will not wake up until C19 reset is issued either via hardware reset or C19 reset control bit. PnP registers and C19 memory will remain intact during this power down.

#### nn62h - Initiate DRAM Download

Cmd Number  written to CMDR by PC:	nn62h   -	where “nn” is the upper byte of the 24-bit wave table DRAM sample memory address where downloaded data is to be placed.

Parameters written to CMDR by PC:	The lower 16-bits of the download address must be supplied as the single parameter to this command when CMD_WE goes ON after loading CMDR with nn62 initially.

Response written to CMDR by C19:	None, at present.

Restrictions:	The product should be configured to support DRAM download. In addition, the DRAM address should be within range (< 24-bits). Finally, the initial value for the programmed I/O download should be pre-loaded into the DATR register, before issuing the nn62 command.

Description:

This command may be used by the PC to place the C19 firmware in a “download” mode and to specify where in sample memory the download should begin.  All downloaded data shall be transferred via the DATR register 16-bits at a time.  Note that this means that 8-bit wave data, must be prepackaged into 16-bit quantities (and sign processed) by the PC software.

Before sending the nn62 command, the PC software should check the DAT_WE bit in STATR, and when it is set, it should then prime the DATR register with the first 16-bit quantity to be downloaded into the sample memory.   After the nn62 command is sent, the C19 uses the “nn” as the upper byte of the 24-bit download address.  The lower 2 bytes of this address are then passed to the C19 via the CMDR parameter.  Once the parameter has been received by the C19, the value in the DATR register is copied to the indicated address in the sample memory.  The C19 then waits for the PC to place another 16-bit quantity in DATR, at which time it copies that value into the next word location in the sample memory.  This process is driven by the event consisting of the PC placing the next 16-bit value into DATR.  It continues indefinitely, until the PC has completed downloading all necessary data, at which time it issues the 63h (terminate download) command to the C19.  The C19 continues accepting 16-bit download values until it receives the 63h command.

Note, it is assumed that the C19 can continue to process DMA wave I/O and MPU-401 MIDI output, while in the download mode.  However, the capabilities of doing simultaneous wave, MIDI, and download operations may be limited by the PC software’s ability to manage these tasks concurrently.

#### 63h - Terminate DRAM Download

Cmd Number  written to CMDR by PC:	63h

Parameters written to CMDR by PC:	None, at present.

Response written to CMDR by C19:	None, at present.

Restrictions:	None, though this command should be ignored by the C19 if received when no download is in progress.

Description:

This command may be used by the PC to instruct the C19 firmware to terminate the  “download” mode.  It is issued by the PC software after the last 16-bit quantity of download data has been placed in the sample memory.

#### 65h -Set Homac Decompression Filter Coefficient

Cmd Number  written to CMDR by PC:	65h

Parameters written to CMDR by PC:	16-bit Homac decompression filter coefficient.

Response written to CMDR by C19:	None, at present.

Restrictions:	Diagnostic.

Description:

This command may be used by the PC to write to the Homac decompression filter coefficient register for tweaking of the Wavetable output sound quality.

## APPENDIX A --	Windows Driver Support for Wave Table DRAM Download

#### DRV_RWA_DOWNLOAD Message

Most of the messages handled by the WaveArtist Windows audio drivers are standard audio driver messages defined by the system.  The normal way for an application program to deal with the driver is via the standard multimedia DLLs, such as provided by MMSYSTEM or MSMIXMGR, who in-turn, generate the required messages sent to the driver.

However, the Wave Table download function is not a standard Windows multimedia function, and some method needed to be devised which allows an application program access to this capability.  For this purpose, the installable interface (DriverProc()) message handler within the WaveArtist driver has been modified to process a new, “user defined” message, as allowed in the Windows documentation.  This message is defined in the C portion of the driver as follows:

#define DRV_RWA_DOWNLOAD    (DRV_USER + 0)

The DRV_USER label is available in MMSYSTEM.H and is set by the system to the value 0x4000; the application should define this message identifier identically to the WaveArtist driver.  The DRV_RWA_DOWNLOAD message is implemented as a “user defined message” which an application can send to the driver by using the “SendDriverMessage()” standard API function call.  To use this message, an application first must call the standard function: OpenDriver() in order to obtain the correct handle for the WaveArtist driver.  It then can use SendDriverMessage() to download DRAM data.  When it has completed its download, it uses CloseDriver() to close off this special link to the driver.

Note that the SendDriverMessage() function has three parameters.  The first, is the driver “handle” which is retrieved by using OpenDriver().  The second and third are optional 32-bit parameters for specific use by the driver.  In this case, the first parameter (dwParam1) is setup as a pointer to the RWA_DLOADINFO structure.  The DRV_RWA_DOWNLOAD message, when processed by the WaveArtist driver always results in one of two DWORD statuses:

1L	The requested download operation has completed successfully.  Note, at present, no ability has been added into the driver to perform a “callback” function to inform the application of success.  Instead, the entire operation is synchronous, in that the application does not receive control until the download is complete (or fails).

0L	The requested download operation has failed.  Some or all of the download data did not get transferred to the hardware.  The hardware may have a problem, or the parameters in the RWA_DLOADINFO structure may be erroneous.

#### RWA_DLOADINFO Structure

The application instructs the driver about specifics for the download operation by passing the driver a pointer to a “RWA_DLOADINFO” data structure, which is defined by the WaveArtist driver as follows:

typedef struct tagRWA_DLOADINFO
{ 

 WORD  dwStructSize 
 WORD  wBitsPerSample;
 LPSTR lpFileToOpen;
 DWORD dwDramAddr;
 DWORD dwDataSize;
 HPSTR hpData

} RWA_DLOADINFO, FAR *LPRWA_DLOADINFO;

This structure must be properly allocated and filled out by the application software before sending the DRV_RWA_DOWNLOAD message. The fields within the RWA_DLOADINFO structure, a pointer to which must be passed to the driver as the first parameter in the SendDriverMessage() call, are utilized by the WaveArtist device driver and the application program as described below:

dwStructSize	This 16-bit field is used as a standard Windows sanity check.  It must be set with the size of the RWA_DLOADINFO structure (for instance using the C language sizeof() function) by the application program before sending the download message.  This runtime value must match the WaveArtist’s own structure size for RWA_DLOADINFO, or the driver returns a 0L (failure) for the SendDriverMessage() function call.

wBitsPerSample	This 16-bit  field is used by application if and only if the download operation is NOT for a standard RIFF format wave file.  When a wave file is downloaded, the driver reads the sample width from the wave file header, and wBitsPerSample is ignored.  If the file is a non-wave file however, or if the download operation is for a data buffer (lpFileToOpen is NULL) then wBitsPerSample is used by the driver to determine how to preprocess the data being downloaded.  The WaveArtist driver supports only two values in this case:

16 (16-bit data, the default) --	In this case, the driver will assume that all data passed to the WaveArtist hardware, is already properly formatted as 16-bit quantities.  If this is a buffer download, it is assumed that the dwDataSize parameter, which is always a “byte count” must be even, implying that dwDataSize/2 is the number of 16-bit words to be sent.  In this case, the hpData parameter must point to the first 16-bit quantity. If these conditions are not met (for instance if dwDataSize is odd while wBitsPerSample is 16) then the driver returns a failure status (0L).  If this is not a buffer download, but rather, a binary (non-wave) file download, the contents of the file will be treated as 16-bit word quantities.  In this case the file will be broken into 64K chunks of download buffers (see lpFileToOpen description below).  It is possible that the final chunk of this file will have an odd number of bytes.  If wBitsPerSample is 16 in this situation, the driver will return a failed status after downloading all previous file chunks, since only the final chunk will have the odd size.

8 (8-bit data) --	In this case, the driver will assume that all data bytes passed to the WaveArtist hardware must be sign converted.  The dwDataSize parameter may be odd or even -- but if it is odd an extra byte of  silence is padded since all transfers are still done 16-bits at a shot, through the DATR register.  The sign conversion is applied to each byte of each 16-bit transfer.  The value 0x80 is added (without carry) to each byte.

lpFileToOpen	The application sets this field to NULL if it wishes to request a download buffer operation.  In that case, the hpData parameter is used by the driver to download data already present in PC memory.  If however the lpFileToOpen field is not NULL, it is assumed to point to a zero terminated ASCII file name.  In this case, the driver attempts to open this file, first as a RIFF wave file, and if that fails, as a binary file.  If the file open fails anywhere along the line, the SendDriverMessage() return value is 0L.  If a wave file is found, the wave header information is used to determine if the data is to be treated as 8-bit or 16-bit, and the driver processes the data as described under wBitsPerSample above, although the wBitsPerSample field is ignored.  If this is not a wave file, the wBitsPerSample field is used.  In either case, the driver attempts to download the data read from the file in 64K chunks (except for the last chunk).  This allows very large files to be downloaded by the driver without needing to allocate more than 64K of memory.  For 8-bit data, the final chunk may be padded with an extra silence byte.  Note also, that the final chunk is sized to whatever data is left in the file, and is normally less than 64K bytes long.

dwDramAddr	This 32-bit field is used by the driver in constructing the nn62 “begin download” command to the WaveArtist hardware.  It specifies a 24-bit sample memory address where data is to be downloaded.  It must be an even word address which falls within the 24-bit range, or a failure return is generated.

dwDataSize	This 32-bit field is the number of 8-bit bytes to be transferred in a buffer download request.  It is ignored if this is a file download request.  But if lpFileToOpen is NULL, then dwDataSize must be supplied.  If wBitsPerSample is 8, then dwDataSize may be odd or even, but if this is specified as 16-bit data, the driver checks that the size is an even number of bytes.  Also, a failure return is generated if “dwDataSize+dwDramAddr” would overflow the sample memory -- the requested download buffer should fit.

hpData	If this is a buffer download, the hpData field is assumed to be a “huge” pointer to the data being downloaded.  This allows the data buffer to exceed 64K bytes in length if desired.  For file downloads, this field is ignored by the driver.

#### File Download Example

The following example code shows how the DRV_RWA_DOWNLOAD message may be used by a C language application.  Hopefully, other language examples will be intuitive.

#define DRV_RWA_DOWNLOAD (DRV_USER+0)

struct 
   { 

     WORD  dwStructSize 
     WORD  wBitsPerSample;
     LPSTR lpFileToOpen;
     DWORD dwDramAddr;
     DWORD dwDataSize;
     HPSTR hpData

   } rwaDlInfo;

HDRVR rwaHANDLE;

DWORD dwDramAddr = 0;

int iCnt;

if ((rwaHANDLE = OpenDriver( "c:\\wac\\rwa.drv", NULL, NULL )) != NULL)

{

rwaDlInfo.dwStructSize = sizeof(rwaDlInfo);

rwaDlInfo.lpFileToOpen = (LPSTR) “TEST.WAV”;

rwaDlInfo.dwDramAddr =   dwDramAddr;

//would be used by driver only if not a wave file

rwaDlInfo.wBitsPerSample = 16;

rwaDlInfo.HpData = (HPSTR) NULL;

//download file TEST.WAV to WaveArtist DRAM

if (!SendDriverMessage(  rwaHANDLE,

DRV_RWA_DOWNLOAD,

(DWORD) (char _far *) &rwaDlInfo,

(DWORD) NULL)

)

MessageBox( hWnd,

szDLFile,

"Unable to Download file.",

MB_OK|MB_APPLMODAL|MB_ICONSTOP

);

//now close down direct link to driver

CloseDriver (rwaHANDLE, NULL, NULL);

}

}